helios_postprocessor

User Guide for Helios Simulations

Version 3.0   |   April 2026

github.com/tamehlhorn/helios_postprocessor

# 1.  Overview

helios_postprocessor is a Python package for post-processing 1D spherical ICF implosion simulations produced by the Helios radiation-hydrodynamics code. It reads EXODUS (.exo) output files and generates publication-quality diagnostic reports, text summaries, time-history CSV files, and quantitative comparisons with published target designs.

Key capabilities:

- Extracts 60+ physics quantities: drive characterization, implosion dynamics, stagnation, burn metrics, shock diagnostics, and adiabat history

- Generates a multi-page PDF diagnostic report automatically

- Compares simulation results against published values via a simple JSON configuration file

- Supports 1D spherical geometry with 3- and 4-region target structures (hot spot, DT ice, foam ablator, outer ablator)

- Works with PDD (Polar Direct Drive) and HDD (Symmetric Direct Drive) configurations

# 2.  Installation

## 2.1  Requirements

Python 3.8 or later. The following packages are required:

| Package | Purpose |
| --- | --- |
| numpy | Array operations |
| scipy | Signal processing, interpolation |
| matplotlib | Plot generation |
| netCDF4 | Reading EXODUS (.exo) files |
| scikit-learn | Optional: RANSAC shock fitting |

## 2.2  Installation Steps

Clone the repository and install in editable mode:

git clone https://github.com/tamehlhorn/helios_postprocessor.git

cd helios_postprocessor

pip install -e .                    # Mac Studio / Linux

pip install -e . --user             # MacBook with Anaconda

Verify the installation:

python3 -c "import helios_postprocess; print('OK')"

# 3.  Quick Start

The single-command runner handles the entire analysis pipeline:

python3 ~/helios_postprocessor/examples/run_analysis.py <base_path>

Where <base_path> is the simulation path without file extension. For example:

python3 ~/helios_postprocessor/examples/run_analysis.py \

~/Sims/MyTarget/MyTarget_01/MyTarget_01

The script automatically finds all input files and generates all outputs:

| File | Direction | Description |
| --- | --- | --- |
| <base>.exo | Input (required) | EXODUS simulation output from Helios |
| <base>.rhw | Input (optional) | Helios RHW input file — enables laser config parsing |
| <base>_published.json | Input (optional) | Published target values for comparison |
| <base>_report.pdf | Output | Multi-page diagnostic plots (20+ pages) |
| <base>_comparison.pdf | Output | Comparison table with published data |
| <base>_summary.txt | Output | Text summary with all metrics |
| <base>_history.csv | Output | Time-history data for post-processing |

To include contour plots (disabled by default for speed):

python3 ~/helios_postprocessor/examples/run_analysis.py <base_path> --contours

# 4.  Target Structure Requirements

The postprocessor reads region structure automatically from the EXODUS file. Region interface node indices are used to identify zone boundaries between physical layers. The supported structures are:

| Region | Material | Role |
| --- | --- | --- |
| 1 | DT Vapor / Gas | Hot spot |
| 2 | DT Ice / DT Solid | Cold cryo fuel — sets adiabat |
| 3 | DT-CH foam / Ablated DT | Ablator / mixed layer |
| 4 | CH Skin / CD ablator | Outer ablator |

Note: 3-region targets (no separate DT ice layer) are supported but the adiabat diagnostic is not valid for DT-CH foam mixed materials. A pure DT ice region (Region 2) is required for the adiabat calculation to be physically meaningful.

# 5.  Output Metrics

## 5.1  Timing

- Stagnation time: minimum hot-spot outer radius (not peak density)

- Bang time: peak DT fusion rate

- Burn width: FWHM of fusion power history

## 5.2  Implosion

- Peak implosion velocity: maximum inward shell velocity, evaluated in shell zones only, strictly pre-bang time

- Mass-averaged adiabat: α = P / P_Fermi where P_Fermi = 2.17(ρ/ρ₀)^(5/3) Mbar, ρ₀ = 0.205 g/cc (Lindl DT ice convention). Evaluated at peak velocity in cold DT ice zones with ρ > ρ₀ to exclude hot-spot gas

- IFAR: R_shell / ΔR at peak velocity, shell boundaries from density profile using ρ > ρ_peak/e threshold

- Hydrodynamic efficiency: max(KE_inward) / E_absorbed

- Stagnation convergence ratio: R₀ / R_hs at stagnation

- In-flight convergence ratio: R₀ / R_ablation-front at peak velocity

## 5.3  Burn and Yield

- Fusion yield: computed from Helios time-integrated neutron count × 17.6 MeV (not from EXODUS burn rate)

- Neutron-averaged temperature, pressure, and areal density

- Burn-averaged quantities (weighted by ρ²σv)

- Ignition criterion: hot-spot ρR ≥ 0.3 g/cm² (Olson et al. convention)

## 5.4  Shock Diagnostics

- First shock in DT ice: tracked using pressure gradient detection

- Shock foot pressure: peak post-shock pressure in DT ice during foot pulse

- Shock breakout: time, pressure, and Mach number when shock reaches hot-spot boundary

- Rankine-Hugoniot: Mach number and density jump from pressure ratio (γ = 5/3)

# 6.  Comparing with Published Data

Place a JSON file named <base>_published.json next to the .exo file. Each entry is [value, uncertainty]. Entries set to [0.0, 0.0] are skipped in the comparison table.

Example JSON for a 4 MJ HDD target:

{

"laser_energy_MJ":       4.0,

"peak_velocity_kms":     [410, 0.0],

"adiabat":               [6.0, 0.0],

"ifar":                  [20.0, 0.0],

"hydro_efficiency_pct":  [8.0, 0.0],

"inflight_KE_kJ":        [300.0, 0.0],

"fraction_absorbed_pct": [97.0, 0.0],

"imploded_DT_mass_mg":   [3.0, 0.0],

"T_hs":                  [46.7, 4.8],

"P_hs":                  [2720, 212],

"rhoR_cf":               [1.60, 0.46],

"CR_max":                [20.1, 9.6],

"yield":                 [256.0, 0.6],

"gain":                  [65.0, 0.0]

}

The comparison table appears in the console output, the _summary.txt file, and a dedicated _comparison.pdf with color-coded cells (green < 20%, yellow 20-50%, red > 50% deviation).

# 7.  Key Physics Conventions

| Quantity | Internal Unit | Notes |
| --- | --- | --- |
| Temperature | eV (arrays), keV (scalars) | Helios stores eV; divide by 1000 |
| Pressure | J/cm³ (arrays), Gbar (output) | Multiply by 1e-8 for Gbar |
| Velocity | cm/s (arrays), km/s (output) | Multiply by 1e-5 for km/s |
| Areal density | g/cm² | ρR = ∫ρ dr (cumulative line integral) |
| Laser energy | MJ | Multiply by 1e-6 from Joules |
| Time | ns (output) | Helios stores seconds; multiply by 1e9 |
| Pressure (3-component) | ion + rad | rad_pressure = elec + actual_rad |

Important: EXODUS files contain only a fraction of actual simulation timesteps. Helios time-integrated quantities (neutron count, deposited energy) are authoritative. EXODUS burn rates are used only as weighting functions for burn-averaging, never for absolute yield.

# 8.  Additional Diagnostic Scripts

## 8.1  Adiabat History

Computes the mass-averaged fuel adiabat at every timestep, showing how entropy is deposited during the foot pulse and main drive:

python3 plot_adiabat_shock.py <base_path>

Produces <base>_adiabat_history.pdf and <base>_shock_history.pdf alongside the .exo file. Requires a 4-region target with a distinct DT ice layer.

## 8.2  Standalone Module Usage

The physics modules can also be used directly in Python scripts:

from helios_postprocess.adiabat_history import (

compute_adiabat_history, print_adiabat_history)

from helios_postprocess.pressure_gradients import analyze_first_shock

from helios_postprocess.burn_averaged_metrics import (

extract_histories_from_run_data,

calculate_burn_averaged_metrics)

# 9.  Known Limitations

- 1D model: Helios cannot capture 3D geometric effects such as beam miss in PDD geometry. For NIF PDD targets, Helios typically over-drives by 10-40% compared to 3D codes (HYDRA/LILAC/Xrage). The absorbed fraction is insensitive to cone angle and spot size.

- Adiabat: The Lindl DT ice convention (ρ₀ = 0.205 g/cc) is not valid for DT-CH mixed foam ablators. Set adiabat = [0.0, 0.0] in the JSON for 3-region foam targets.

- IFAR: The density-based shell finder may misidentify the shell in foam ablator targets with gradual density profiles. Published IFAR values often use different definitions (e.g. ablation front at 2/3 initial vapor radius).

- CR definition: The postprocessor uses R₀/R_hs at stagnation (hot-spot boundary from region interfaces). Some published values use shell mid-radius or a temperature mask at ignition.

- Shock transit: EXODUS sampling (typically 0.1-0.25 ns) may be too coarse to resolve the foot shock transit through the DT ice. Use 0.05 ns output intervals during the foot phase to capture this.

- Burn-averaged quantities: Weighted by ρ²σv from EXODUS-sampled hot-spot profiles. Accuracy improves with finer output intervals during the burn (0.005-0.01 ns recommended).

# 10.  Repository Structure

| File | Role |
| --- | --- |
| helios_postprocess/core.py | EXODUS/netCDF4 reader |
| helios_postprocess/data_builder.py | Builds ICFRunData dataclass with unit conversions |
| helios_postprocess/icf_analysis.py | ICFAnalyzer: all physics calculations |
| helios_postprocess/icf_plotting.py | ICFPlotter: multi-page PDF report |
| helios_postprocess/icf_output.py | Text summary and CSV time histories |
| helios_postprocess/burn_averaged_metrics.py | Burn-averaging and published-data comparison |
| helios_postprocess/adiabat_history.py | Adiabat history module |
| helios_postprocess/pressure_gradients.py | Shock identification and first-shock analysis |
| helios_postprocess/energetics.py | Kinetic energy, hydro efficiency, PdV work |
| helios_postprocess/rhw_parser.py | Helios RHW input file parser |
| examples/run_analysis.py | Single-command CLI runner |
| plot_adiabat_shock.py | Standalone adiabat and shock diagnostic plots |

Xcimer Energy — Target Design Group   |   Contact: Tom Mehlhorn