NEUTRONICS FINDINGS NOTE

DSR, Areal Density, and Yield Unfold: OpenMC Transport vs NeSST on N210808

For: Kyle Keipper, Will
From: Prof. Tom Mehlhorn (MECS)
Run: 090425_KK_opt_2burn (N210808 clean-1D burn), bang-time snapshot, ENDF/B-VIII.0, 400k particles, neutron + photon transport
Date: 2026-07-30, updated 2026-07-31 (added the self-consistent first-principles coefficient and the final pipeline DSR after the traversed-ρR switch)

Bottom line

Three methods on the same Helios snapshot agree once the areal-density definition is pinned down. The OpenMC / NeSST “discrepancy” was the Method-1 vs Method-2 distinction from Kyle’s slide, now resolved and wired into the pipeline: run_analysis STEP 5b now reports DSR 4.52% (fuel) / 5.02% (+carbon) on the emission-weighted (traversed) ρR, matching OpenMC (4.3%). A first-principles reproduction of the NIF yield unfold shows the standard 4·DSR exponent is design-specific (~5 for this HDC-ablator target).

1.  Two areal densities

| quantity | OpenMC (transport) | NeSST (as shipped) | analytic Method-2 |
| --- | --- | --- | --- |
| ρRD (g/cm²) | 0.372 | 0.567 | 0.297 |
| DSR (10–12 / 13–15) | 4.31% | 10.07% (fuel 9.51%) | — |

NeSST ρRemission = center-to-edge ∫₀R ρ dr (Kyle Method 1). OpenMC and the analytic Method-2 (isotropic 4π angle-averaged chord integral, validated: central source → ρ·R, uniform → 0.75×) give the emission-weighted traversed ρR. Both traversed values (0.30–0.37) are ~half the center-to-edge (0.57); DSR follows (4.3% vs 10%). Abu-Shawareb: “the areal density of the system is emission weighted to larger radii, which is why…the areal density decreases as the yield increases” — so DSR tracks the traversed ρR (Method 2), not center-to-edge. Residual OpenMC (0.372) vs Method-2 (0.297) is multiple-scatter + energy-dependent σ (transport-only).

Pipeline run (STEP 5b, traversed mode) on the same snapshot: traversed ρR D = 0.298, T = 0.447, C = 0.664 g/cm² (fuel D+T = 0.745, total = 1.409); DSR 4.52% fuel / 5.02% total. The pipeline traversed ρRD (0.298) reproduces the analytic Method-2 (0.297) to 1%.

Action: NeSST is unchanged (a pure ρR → DSR mapper). We changed our code: _composition_scatter now feeds it the traversed ρR (new target_composition.traversed_species_areal_densities); center-to-edge is retained and selectable via rhoR_mode.

1a.  First-principles DSR ↔ ρR coefficient (self-consistent)

The empirical NIF calibration is ρR = 20.4 × DSR (g/cm² per unit DSR). Our first-principles value must use the same areal-density convention on both sides. Using the traversed (Method-2) ρR that the DSR is now derived from:

| coefficient | value (g/cm² per DSR) | definition |
| --- | --- | --- |
| total (D+T+C) | 28.1 | total traversed ρR / total DSR (1.409 / 0.0502) |
| fuel (D+T) | 16.5 | fuel traversed ρR / fuel DSR (0.745 / 0.0452) |

These bracket the empirical 20.4, as expected for a clean over-igniting 1-D burn where ρR/DSR run high. Reporting fix: the pipeline previously printed a single “31.4” that mixed conventions — center-to-edge fuel ρR (1.42) over the traversed fuel DSR (0.0452). That number was neither Method 1 nor Method 2 and has been corrected to the two self-consistent values above (neutron_scatter now uses the traversed convention on both sides). The empirical relation is tied to the total, emission-weighted ρR, so 28.1 is the value to compare against 20.4; the fuel-only 16.5 is the internal D+T check.

2.  First-principles yield unfold   Y_total = Y(13–15) e^(4·DSR)

| quantity | value |
| --- | --- |
| escaping 13–15 MeV / born | 0.786 |
| Y_total / Y(13–15), OpenMC (1/N) | 1.272 |
| paper e^(4·DSR) | 1.188 |
| effective exponent k | 5.6 (~5.1–5.2 after DD/TT correction) |

k ≫ 4 because n–12C elastic loses little energy per collision, pushing 14 MeV neutrons into 12–13 MeV (out of the unscattered window, above the 10–12 DSR window), which inflates scattered-out / DSR. Consequence: the standard 4·DSR unfold under-estimates total yield by ~7% (1.19 vs 1.27) for a heavy-ablator target. A DT-only source run would pin the exponent exactly.

3.  Gamma signature

0.0070 photons / source neutron escaping, ~540:1 dominated by 12C(n,n′γ) 4.4 MeV (6.8×10-3) over H(n,γ) 2.2 MeV (1.3×10-5). The 4.4 MeV line is a clean HDC-ablator carbon signature.

Caveats

Clean-1D burn (Y ~ 15.6 MJ vs 1.37 measured); ρR/DSR run high vs the real shot (published DSR 2.87%). This demonstrates the method + corrections, not an experimental match. One snapshot. Surface-tracking warnings are negligible (DSR stable to ~3% across two geometry rebuilds). σel,D = 0.641 b from ENDF/B-VIII.0 at 14.06 MeV.

Recommendations

- Use the emission-weighted (Method-2 / OpenMC) ρR against the measured DSR (pipeline now default).

- Don’t assume a 4·DSR unfold for carbon ablators; derive it per design (~5 here).

- Pin which ρR the experimental DSR ↔ ρR calibration is tied to (the emission-weighted total, per the 28.1 vs 20.4 comparison above).

- Next: a degraded / under-igniting N210808 (lower ρR) to map how the Method-1/2 gap, the unfold exponent, and the DSR ↔ ρR coefficient scale with yield.

Reproduce: examples/dump_snapshot.py (Helios env) → examples/compare_openmc_nesst.py (openmc-env).