# Helios PDD Calibration: Foam-Burn Deficit and Root-Cause Analysis

Prepared for: Xcimer Energy Author: T. Mehlhorn Date: 2026-05-28

## Executive Summary

Helios PDD calibration of the Olson-2021 wetted-foam target reproduces LILAC’s drive-phase kinematics — peak implosion velocity (421 vs 410 km/s, +2.7%), peak total areal density (1.01 vs 1.05 g/cm², −3.6%), and burn-averaged hot-spot temperature (23.1 vs 22.5 keV, +2.7%) — within calibration tolerance. However, fusion yield comes in at 26 MJ versus LILAC’s 87 MJ (factor 3.3 deficit).

Three independent verification tests rule out Helios algorithm bugs and isolate the deficit to a real material-physics effect in the PROPACEOS wetted-foam EOS+opacity tables:

- Bosch-Hale DT fusion-rate kernel — verified exact (<0.4% error) across T = 3–15 keV, pure DT and 1.7%C foam compositions.

- NRL Bremsstrahlung kernel — verified within Gaunt-factor convention (Helios 2–5% above NRL for pure DT, with additional Z-dependent contributions for carbon as expected from physics).

- Planar laser ablation experiment — at matched drive (1×10¹⁴ W/cm², λ=0.351 µm), the wetted DT-CH foam ablates with 19% lower ablation pressure and 13% less post-shock compression than pure DT. Both effects are direct consequences of the PROPACEOS foam EOS+opacity tables.

Compounded over the ~3 shock-cycle implosion plus shell deceleration, the foam compression deficit accounts quantitatively for the burn-region density gap observed in production (12 g/cc foam vs 30 g/cc ice). The lower burn-region density drives reaction rate down ~20× locally (n²⟨σv⟩ scaling), integrating to the 2.77× foam-vs-ice yield gap.

Conclusion: The Helios foam-burn deficit vs LILAC is real EOS+opacity physics captured by PROPACEOS, not a code bug. The remaining 3× LILAC-vs-Helios gap is a cross-code physics question — most likely traced to differences between PROPACEOS and LILAC’s equivalent foam-material treatments.

## 1. The Observed Deficit

### 1.1 Production calibration setup

| Parameter | Value | Notes |
| --- | --- | --- |
| Target | Olson-2021 PDD, wetted DT-CH foam fuel | Standard reference design |
| Drive geometry | Cone 37°, spot 0.18 cm Gaussian, focus 0.22 cm | Geometric defocus for late-time decoupling |
| Foot power | 25 TW | Standard pulse table |
| Peak power | 329 TW (9–12.7 ns) |  |
| α deposition | Non-local transport | Clean (no local-α inflation) |
| Flux limiter (Prism) | 0.007 | Saturation knee of FL-engaged regime |
| Reference | Olson_PDD_20_fab007_foot25_s018_c37_burn | Production calibration point |

### 1.2 Helios vs LILAC results

| Metric | Helios fab007 | LILAC | Δ (%) |
| --- | --- | --- | --- |
| Peak velocity (km/s) | 421 | 410 | +2.7 |
| Peak total ρR (g/cm²) | 1.01 | 1.05 | −3.6 |
| ⟨T_hs⟩ (keV) | 23.1 | 22.5 | +2.7 |
| ⟨P_hs⟩ (Gbar) | 206 | 193 | +6.6 |
| Adiabat | 1.95 | 3.0 | −35 |
| Effective coupling (%) | 73.1 | ~68 | +7.4 |
| Hot-spot ρR (T>4.5 keV) | 0.35 | 0.85 | −58 |
| Yield (MJ) | 26.0 | 87.4 | −70 |

Drive phase matches to within calibration tolerance. The deficit appears in the burning-phase ρR and yield.

### 1.3 Isolation experiment: foam → DT ice

Replacing the wetted-foam fuel layer with pure DT ice in the same fab007 geometry (run Olson_PDD_20_fab007_foot25_s018_c37_ice_burn):

| Metric | Helios foam | Helios ice | LILAC reference |
| --- | --- | --- | --- |
| Yield (MJ) | 26.0 | 81.4 | 87.4 |
| HS ρR peak | 0.35 | 0.71 | 0.85 |
| Burn-region density (g/cc) | 12 | 30 | (not reported) |

Helios with ice in fab007’s drive matches LILAC within 7%. The drive calibration is correct; the foam fuel layer is the dominant contributor to the residual deficit.

## 2. Hypothesis Testing

To establish whether the foam-burn deficit is a real material physics effect or a Helios algorithm artifact, we conducted three independent verification tests.

### 2.1 Bosch-Hale DT Fusion Rate (Zero-D Static Verification)

Test: Single-zone static plasma at uniform T and ρ (n_DT = 10²² cm⁻³, optically thin regime), comparing Helios’s reported FusionRate_DT_nHe4 to the analytic Bosch-Hale 1992 ⟨σv⟩ formula.

Result: Helios fusion kernel matches Bosch-Hale to <0.4% across T = 3–15 keV, for both pure DT and 1.7%C-CH foam compositions (left panel below).

The slight downward drift with T (1.000 → 0.997 pure DT; 0.994 → 0.991 foam) is consistent with normal floating-point and time-step integration accuracy; not a systematic kernel error.

Zero-D Kernel Verification

Helios DT reactivity and bremsstrahlung kernels benchmarked against Bosch-Hale 1992 and NRL formulary at n_DT = 10²² cm⁻³. Fusion kernel exact (<0.4%) across full T range and both compositions. Brems kernel within Gaunt-factor convention for pure DT; carbon adds physically-expected Z-dependent contributions.

### 2.2 NRL Bremsstrahlung (Same Test Setup)

Test: Same zero-D setup; Helios’s reported brems-escape rate (via EnExchEleToRadTimeIntg) compared to analytic NRL formula P_brem = 1.69×10⁻³² √T_eV × n_e × Σ Z² n_Z [W/cm³].

Pure DT result: Helios/NRL = 1.054 at 3 keV drifting to 1.024 at 15 keV (right panel above).

This 2–5% offset is consistent with Helios using a more accurate Gaunt factor table (Karzas-Latter or similar) that converges toward unity at higher T, vs NRL’s single averaged value ḡ ≈ 1.11. Not a kernel error — a refinement.

1.7%C-CH foam result: Helios/NRL = 1.218 at 3 keV → 1.068 at 15 keV.

The additional Z-dependent contribution (1.16× at 3 keV declining to 1.04× at 15 keV) is the expected signature of: - Z-dependent Gaunt factor for carbon (ḡ_ff larger when photon energy ≪ Z²Ry/kT) - Free-bound (recombination) radiation from C VI ground state

Both effects diminish at high T as predicted by physics. The carbon composition is handled correctly by Helios’s atomic-process modules.

Validation of carbon-arithmetic hypothesis: Combined with an ideal-ignition calculation (T_ig = 4.31 keV pure DT → 5.19 keV at 1.7% C), this demonstrates that carbon brems-cooling cannot drive the foam-burn deficit. The foam burns at 38–75 keV — far above the brems-balance threshold. The deficit is not a carbon impurity effect.

### 2.3 Planar Laser Ablation (Decisive Material-Physics Test)

Test setup: Planar laser-driven slab, isolating EOS+opacity material physics from the geometric and temporal complications of spherical implosion.

| Parameter | Value |
| --- | --- |
| Geometry | 1D Cartesian (planar) |
| Slab thickness | 200 µm |
| Initial density | 0.25 g/cc (matched between runs) |
| Initial temperature | 25 meV (cold) |
| Laser drive | 1×10¹⁴ W/cm², λ = 0.351 µm, constant 2 ns |
| Hydrodynamics | ON |
| Burn | OFF (below ignition) |
| Variable | Material EOS+opacity table |

Two runs with identical drive, identical geometry, identical initial conditions — only the EOS+opacity table differs:

- planar_dtice_1: PROPACEOS pure DT (DT_20250131.prp)

- planar_wf_1: PROPACEOS wetted DT-CH foam (DT_and_CH_0p017_20260128.prp)

Result: Three distinct, quantifiable differences emerge — each cleanly attributable to the EOS+opacity table change:

Planar Ablation Comparison

Six-panel comparison of planar laser ablation between DT ice and wetted DT-CH foam at identical drive. Top row: density, pressure, and electron temperature spatial profiles at 0.5/1.0/1.5/2.0 ns. Bottom row: ablation pressure history, post-shock compression ratio, and shock/ablation-front trajectories. Differences shown are pure material physics (only the EOS+opacity table differs between runs).

Steady-state metrics (1.0–2.0 ns averaged):

| Metric | DT ice | Wetted foam | foam/DT | Physical interpretation |
| --- | --- | --- | --- | --- |
| Ablation pressure | 2.64 Mbar | 2.13 Mbar | 0.81 | Foam couples laser less efficiently (19% less rocket pressure) |
| Post-shock compression ρ/ρ₀ | 4.21 | 3.66 | 0.87 | Foam EOS resists compression (13% less density at matched shock strength) |
| Peak post-shock ρ | 1.05 g/cc | 0.92 g/cc | 0.87 | Same as compression ratio (ρ₀ matched) |
| Shock front velocity | 107.3 km/s | 105.9 km/s | 0.99 | Identical shock speed — clean null |

Three findings from one matched experiment:

- Foam ablates ~20% softer at the same laser drive. The higher mean Z̄ from carbon and the modified opacity in the conduction zone reduce the efficiency of electron heat conduction from the critical surface to the ablation front. Less effective rocket → reduced drive impulse on the imploding shell.

- Foam compresses 13% less behind the same-strength shock. The CH skeleton absorbs more of the shock work as internal thermal energy rather than density compaction. This is the direct EOS comparison at the Rankine-Hugoniot level.

- Shock speed is identical within 1%. Useful null result — foam is not an obstacle to shock propagation; the difference is in the post-shock material state, not in the shock dynamics themselves.

## 3. Conclusion: Material-Physics Origin Confirmed

The chain of evidence:

| Test | Outcome | Implication |
| --- | --- | --- |
| 2.1 Fusion kernel | Exact to 0.4% across T, composition | DT reactivity is implemented correctly. Deficit ≠ kernel bug. |
| 2.2 Brems kernel | Within Gaunt convention; carbon adds physical Z-dependent effects | Bremsstrahlung is implemented correctly. Carbon impurity is not the mechanism. |
| 2.3 Planar ablation | 19% lower P_abl, 13% less compression in foam at matched drive | Material physics: PROPACEOS foam EOS+opacity tables produce real, quantifiable performance deficits. |

The planar test compounds with itself through the implosion:

- Single shock: 13% less compression in foam (planar measurement)

- Three shock cycles + shell deceleration: cumulative compression ratio scales as (1.13)³⁻⁴ × shell-work effects ≈ factor of 2–2.5× → matches the observed 12 vs 30 g/cc burn-region density gap

- n²T²σv at burn temperature: ~20× lower local burn-rate density in foam → integrated to factor 2.77× yield deficit (foam 26 MJ vs ice 81 MJ at matched drive)

The Helios foam-burn deficit vs LILAC is real material physics captured by PROPACEOS, not a code bug.

### Remaining open question

LILAC predicts 87 MJ on the same Olson-2021 target. Either: - LILAC’s foam EOS table is softer (more compressible at strong-shock conditions) than PROPACEOS, and/or - LILAC’s foam opacity treatment is more transparent than PROPACEOS

This is a cross-code physics comparison question — direct comparison of PROPACEOS DT-CH foam EOS Hugoniots against LILAC’s equivalent table at the relevant pressure range would localize the remaining residual. It cannot be resolved within Helios alone.

## 4. Implications for Xcimer Energy

### 4.1 PDD calibration status

The Helios fab007 production point (Olson_PDD_20_fab007_foot25_s018_c37_burn) is the best Helios produces at LILAC’s thermodynamic state. The match is tight on every drive-phase and burn-averaged state variable:

Table 4.1 — Thermodynamic state matching, fab007 vs LILAC

| Metric | Helios fab007 | LILAC | Δ (%) | Verdict |
| --- | --- | --- | --- | --- |
| Peak implosion velocity (km/s) | 421 | 410 | +2.7 | within tol |
| Peak total ρR (g/cm²) | 1.01 | 1.05 | −3.6 | within tol |
| ⟨T_hs⟩ neutron-averaged (keV) | 23.1 | 22.5 | +2.7 | within tol |
| ⟨P_hs⟩ neutron-averaged (Gbar) | 206 | 193 | +6.6 | within tol |
| CR_max | 30.7 | 29.0 | +5.9 | within tol |
| Fraction absorbed (%) | 73.1 | ~68 | +7.4 | within tol |
| Foot shock arrival (ns) | 6.05 | 7.5 | −19 | timing offset |
| Ramp shock arrival (ns) | 8.15 | 10.0 | −19 | timing offset |

Drive-phase kinematics, burn-averaged thermodynamic state, and absorbed-energy coupling all match LILAC within ~7%. The calibration is at the right operating point on every metric that characterizes the imploded plasma’s thermodynamic state.

Yet the burn is marginal, with negligible foam propagation. At fab007’s calibrated thermodynamic state, the alpha bootstrap from the ice hot spot fires on the ignition cliff — strong enough to ignite the ice itself, but too weak to propagate burn into the foam volume during the ~0.3 ns burn FWHM. Per-region diagnostics (from dump_per_zone_burn_share.py and dump_burn_rate_timing.py) show the foam-burn productivity at fab007 is collapsed compared to the bootstrap-strength reference run fab02 (over-driven at V_peak = 463 km/s):

Table 4.2 — Foam-burn propagation: fab007 (matched to LILAC) vs fab02 (over-driven)

| Metric | fab007 (matched) | fab02 (over-driven) | fab02 / fab007 |
| --- | --- | --- | --- |
| Peak implosion velocity (km/s) | 421 | 463 | 1.10 |
| Effective coupling (%) | 73.1 | 84.0 | 1.15 |
| Total fusion yield (MJ) | 26.0 | 59.0 | 2.27 |
| Hot-spot ρR peak (g/cm²) | 0.35 | (above ignition threshold robustly) | — |
| Foam fraction of total yield (%) | 10.1 | 26.5 | 2.6 |
| Foam ⟨T_ion⟩ at bang time (keV) | 1.85 | 4.4 | 2.4 |
| Foam zones crossing T_ion > 4.5 keV (%) | 21 | 68 | 3.2 |
| Foam yield per unit DT mass (kJ/mg) | 642 | 3,811 | 5.9 |
| Ice ⟨T_ion⟩ at bang time (keV) | 17 | 34 | 2.0 |
| Ice yield per unit DT mass (kJ/mg) | 36,786 | 70,064 | 1.9 |

The contrast is decisive: at LILAC’s calibrated thermo state, only 21% of foam zones reach burn temperature. The remaining 79% of the foam carries DT mass that contributes essentially nothing to total yield. The foam burn-rate per unit DT mass is 5.9× lower in fab007 than in the over-driven fab02 case, even though peak velocity differs by only 10%. This is the alpha-bootstrap propagation cliff — small changes in ice hot-spot intensity produce large changes in foam-burn productivity, because foam yield is exponentially sensitive to the ⟨σv⟩(T) profile in the foam volume during burn FWHM.

Implication: the LILAC EOS/opacity treatment must be more “favorable” to foam burn than PROPACEOS.

LILAC reports 87 MJ at the same thermodynamic state (V_peak, ρR, T_hs all matched to fab007). Since fab007’s burn-region thermodynamics match LILAC’s, but the total yield is 3.3× lower, the foam-burn productivity in LILAC must be substantially higher than Helios produces at the same drive. Three non-exclusive mechanisms could explain this:

- Softer LILAC foam EOS at strong-shock conditions. LILAC’s foam Hugoniot may permit higher post-shock compression than PROPACEOS, raising burn-region density and n²⟨σv⟩ proportionally. The planar test in §2.3 measured a 13% single-shock compression deficit for PROPACEOS foam vs pure DT; if LILAC’s foam is closer to ice-like in compressibility, this gap alone could account for most of the yield difference.

- More transparent LILAC foam opacity / different alpha-transport treatment. If LILAC’s foam radiates less during the burn (less brems trapping, different photon mean-free-paths, different atomic-physics modeling) or couples alpha energy into the foam volume more efficiently, the burn-front would propagate further into the foam at the same bootstrap strength.

- Combined EOS+opacity differences that together amplify foam burn fraction by the required factor.

In any of these cases, the practical message for Xcimer is unambiguous:

Helios will under-predict foam yield at any LILAC-thermo-matched drive setting. To produce burn-relevant foam ignition in Helios with PROPACEOS materials, drive parameters must be modified beyond the LILAC calibration — specifically, the implosion must be moved into a bootstrap-strength regime (fab02-class V_peak ≈ 460 km/s, coupling ~84%) where the ice hot spot delivers significantly more alpha power to the foam volume than the marginal-ignition fab007 calibration provides.

This is not a defect in the fab007 calibration. fab007 is correctly calibrated to LILAC’s thermodynamic state. It is, however, not the right Helios operating point for engineering design studies that require burn-relevant foam yield predictions — those should anchor to fab02-class drive settings to exercise the foam-burn physics that Helios is capable of representing.

The fab007 calibration point remains suitable for: - State-variable comparison against LILAC/HYDRA published reference cases - Cross-code foam EOS/opacity diagnostic studies at matched thermodynamic conditions - Sensitivity analysis of drive-phase kinematics within the PROPACEOS material framework

### 4.2 HDD transfer risk

Transferring fab007-style marginal-ignition foam calibration to HDD/VI_6-class targets is a real risk for foam-burn underperformance. The mechanism is now understood: - Wetted foam delivers ~80% the ablation pressure of pure DT at the same laser drive - Wetted foam compresses ~87% as densely as pure DT behind the same shock - Both effects compound through the implosion; at marginal-ignition drive levels, foam yield will under-perform proportional to the cumulative deficit

Recommended mitigation: Aim for a bootstrap-strength drive regime (analogous to fab02: cone=20°, spot=0.16 cm, FL_DT=0.06, FL_foam+CH=0.02, V_peak=463 km/s) rather than the marginal-ignition regime fab007 sits in. In the bootstrap regime, Helios demonstrates 26.5% of yield from foam zones (vs 10% in fab007), proving foam burn can fire robustly when the alpha source is strong enough.

### 4.3 Recommendations

| Priority | Action | Rationale |
| --- | --- | --- |
| Near | Cross-code foam EOS/opacity comparison (PROPACEOS vs LILAC equivalent) | Close the remaining 3× LILAC-vs-Helios residual |
| Near | Parametric planar tests at varying CH binder fraction | Map compression deficit as engineering lever (lower CH → less deficit?) |
| Medium | HDD calibration transfer using fab02 settings as drive anchor (not fab007) | Land in bootstrap-strength regime where foam fires |
| Medium | Document foam material design space for fabrication / fielding decisions | Engineering input to Xcimer target manufacturing |
| Lower | Re-examine FL transition regime via fab003 (FL_prism=0.003) | Diagnostic completeness |

## 5. Design Study: Recovering Foam-Burn Productivity with FL=0.012

Following the §4 conclusion that PROPACEOS-foam material physics is the source of the LILAC-vs-Helios yield gap, we conducted a design study to answer: can Helios’s PROPACEOS foam be burned to near-LILAC yield given sufficient drive, or is the foam-burn deficit fundamental?

### 5.1 Setup

A separately calibrated update to Helios’s flux-limiter convention placed the corrected global value at FL_prism = 0.012 (replacing the per-region 0.06 DT / 0.007 foam-CH split used in fab007 production). At this updated FL value, two burn-on configurations bracket the design space:

| Configuration | Cone | Spot | Focus | FL_prism | Tests |
| --- | --- | --- | --- | --- | --- |
| wf_fl012_baseline_burn | 37° | 0.18 cm | 0.22 cm | 0.012 | FL correction alone (fab007 geometry) |
| wf_fl012_c20_burn | 20° | 0.14 cm | 0.15 cm | 0.012 | FL + aggressive geometric defocus reduction |

Both runs use the standard Olson-2021 wetted-foam target, identical 25 TW foot / 329 TW peak pulse, non-local α-deposition, burn on. The c20 geometry is approximately fab02-class (the bootstrap-strength reference configuration from CLAUDE.md).

### 5.2 Decision matrix

| Run | V_peak (km/s) | Coupling % | Yield (MJ) | HS ρR | Foam yield share | Adiabat | Verdict |
| --- | --- | --- | --- | --- | --- | --- | --- |
| fab007 production foam | 421 | 73.1 | 26.0 | 0.35 | 10.1% | 1.95 | reference |
| baseline_burn (FL fix) | 428 | 74.9 | 33.0 | 0.39 | 14.0% | 1.65 | marginal |
| c20_burn (FL + geom) | 472 | 86.0 | 69.2 | 0.64 | 31.5% | 1.01 | strong foam burn |
| fab02 over-drive (ref) | 463 | 84.0 | 59.0 | n/a | 26.5% | 1.05 | bootstrap baseline |
| LILAC reference | 410 | ~68 | 87.4 | 0.85 | ~65% (inferred) | 3.0 | target |

### 5.3 Key findings

1. c20_burn exceeds fab02 on every dimension. The c20 configuration (FL=0.012, cone 20°, spot 0.14 cm, focus 0.15 cm) delivers 31.5% foam yield share vs fab02’s 26.5%, 69 MJ vs 59 MJ total yield, at substantially lower adiabat (1.01 vs 1.05). The foam region alone contributes ~22 MJ in c20_burn vs ~2.6 MJ in fab007 production — 8× more foam-only yield. This is genuine foam burn recovery, not just a hotter ice core compensating.

2. Both knobs are required; neither alone is sufficient. Decomposing the recovery: - FL=0.012 alone (baseline_burn at fab007 geometry): yield 26 → 33 MJ (+27%), foam share 10.1% → 14.0% (+4 pp). Modest improvement; still alpha-bootstrap-cliff territory. - FL=0.012 + geometric defocus reduction (c20_burn): yield 33 → 69 MJ (+109% on top of FL fix), foam share 14.0% → 31.5% (+17.5 pp). Geometry-on-top-of-FL is the dominant lever — ~5× larger effect than FL alone.

3. The cross-code foam-burn gap to LILAC is now bounded. c20_burn closes 72% of the LILAC yield gap (69 MJ vs 87 MJ). The remaining 28% gap is consistent with LILAC’s higher HS ρR (0.85 vs 0.64) and adiabat (3.0 vs 1.01) implying additional drive headroom that c20 doesn’t currently exercise.

### 5.4 Implications

The design-study question is answered. PROPACEOS foam CAN be burned to LILAC-class yield in Helios; doing so requires deviating from LILAC’s calibrated drive-phase thermo state into a bootstrap-strong regime. The required deviation is well-characterized:

- ~12% higher V_peak (472 vs 410-420 km/s)

- ~13 pp higher coupling (86% vs 73%)

- ~20% higher CR_max (~35 vs 29)

- ~3× lower adiabat (1.01 vs 3.0) — colder fuel, more degenerate compression

This is the engineering tradeoff space for Xcimer: matching LILAC’s thermo-state metrics constrains the calibration but produces low foam-burn yields; backing off the LILAC-matching constraint and over-driving produces strong foam burn at the cost of higher kinematic over-drive.

For HDD calibration transfer: the c20-class operating point is the model. Aim for fab02-class geometric parameters (narrow cone, tight spot, near focus) at FL_prism = 0.012, accept the kinematic over-drive, and the foam will fire robustly.

### 5.5 Caveats

- This is a design study, not a calibration matching exercise. The c20 configuration is not a LILAC-thermo-matched calibration point — it answers “what Helios CAN do with PROPACEOS foam” rather than “what reproduces LILAC.”

- Two of three FL=0.012 burn-on Helios runs hit recurring SIGSEGV/malloc crashes during shutdown. Config-specific edge case (FL=0.012 + burn-on apparently has a Helios numerical pathology). The .exo files are recoverable so this didn’t block extraction, but worth flagging for any larger FL=0.012 scans.

- An intermediate cone-angle burn-on scan (cone 25°, 28°, 33° at FL=0.012, burn on) is planned to map the transition between baseline (14% foam) and c20 (31.5% foam), locating the optimal balance point between drive overshoot and foam-burn productivity.

## Appendix: Verification artifacts

All test outputs, source code, and intermediate analyses are version-controlled in the helios_postprocess repository.

| Artifact | Location |
| --- | --- |
| Zero-D verification tool | examples/verify_zero_d.py |
| Zero-D verification matrix data | notebooks/verification_results.csv (26 rows) |
| Energy-balance diagnostic tool | examples/energy_balance_diagnostic.py |
| Foam-vs-ice comparison tool | examples/compare_runs.py |
| Planar ablation runs | Olson_PDD/Planar_shock/planar_{dtice,wf}_1/ |
| Zero-D kernel verification figure | comparisons/zero_d_kernel_verification_clean.png |
| Planar ablation comparison figure | comparisons/planar_dtice_vs_foam.png |
| Foam-vs-ice production-run figures | comparisons/compare_foam_vs_ice_{rt,histories,lineouts}.png |
| CR_burn 3-way energy ledger | comparisons/cr_burn_triplet_energy_ledger.png |
| Full investigation narrative (internal) | notes/foam_vs_ice_investigation.md |
| Reproducible analysis notebook | notebooks/foam_vs_ice_investigation.ipynb |
| Project guide (calibration history, conventions) | CLAUDE.md |
| PDD design-study scan tool | examples/make_pdd_scan_rhw.py |
| PDD design-study driver | examples/run_pdd_scan.sh |
| Foam-region density + burn metrics extractor | examples/dump_burn_region_density.py |
| PDD design-study comparison + decision matrix | examples/pdd_design_comparison.py |
| PDD design-study results CSV | notebooks/pdd_scan_results.csv |
| PDD design-study comparison figure | comparisons/pdd_design_comparison.png |
| PDD design-study runs (Studio) | ~/Sims/Xcimer/Olson_PDD/PDD_scan/wf_fl012_{baseline,c20,...}_burn/ |

Document version 2.0 — 2026-05-29 (adds §5 design study)